MICRO FILLER w/ CAPPER – Operation Manual

For helpful how-to videos visit our

YouTube channel - Inlinelabeling1

Follow us on:

Instagram @inlinepackaging

Facebook @inlinepack

# CAUTION!

Persons operating this machinery are reminded to observe their own company safety policies. In addition, the following safety rules should be observed:

- DO NOT REACH INTO THE MACHINE WHILE IT IS IN OPERATION.

- USE ONLY THE CORRECT TOOL FOR THE JOB BEING DONE.

- STAY ALERT, REMEMBER LOCATION OF CONTROL SWITCHES.

MAINTENANCE

The main electric switch supplying power to the machinery should be locked out or disconnected when repairs are performed on this equipment.

Machine should be cleaned and inspected regularly.  All safety switches must be operable, attachments secure and machine free of broken glass and paper.

Do not hand lubricate when the machine is in operation.

Work area should be kept clean and as dry as is practical.

The repair or adjustment of this equipment should be performed only by persons qualified through technical training and ability, as assigned by your company.

OPERATION

All guards should be securely in place before operating the machine. Close all doors when operating machine.

Company rules on eye protection should be followed.

Loose clothing or jewelry such as neckties, rolled sleeves, over blouses, bracelets, watches and rings should not be worn when operating the machine.

Report all malfunctions, unusual operation, and defects immediately.

Please exercise caution with any moving parts, including the conveyor and any pinch or drive rolls.

Stop the machine before placing hand or arms near or into any area where moving parts are located.

# TABLE OF CONTENTS

SECTION ONE – GENERAL INFORMATION

| 1.1 | Terminology of Machine | pg. 4 |
| --- | --- | --- |
| 1.2 | Specifications and Requirements | pg. 5 |
| 1.3 | Functional Description of Machine | pg. 6 |

SECTION TWO – UNCRATING AND INSTALLATION

| 2.1 | Electrical | pg. 16 |
| --- | --- | --- |
| 2.2 | Communication Cables | pg. 16 |
| 2.3 | Pneumatic | pg. 17 |
| 2.4 | Installing in Production Line | pg. 18 |
| 2.5 | Leveling Base of Machine | pg. 18 |

SECTION THREE – OPERATIONAL ADJUSTMENTS

| 3.1 | Adjusting Fill Levels | pg. 19 |
| --- | --- | --- |
| 3.2 | Foam Issues | pg. 19 |

SECTION FOUR – PERIODIC MAINTENANCE, CLEANING, AND LUBRICATION

| 4.1 | Maintenance | pg. 20 |
| --- | --- | --- |
| 4.2 | Cleaning the Machine | pg. 20 |
| 4.3 | Lubrication | pg. 20 |

SECTION FIVE – TROUBLESHOOTING

| 5.1 | Nothing Works at All | pg. 21 |
| --- | --- | --- |
| 5.2 | Operational Inconsistencies | pg. 21 |

SECTION SIX – SCREENS

| 6.1 | Main Screen | pg. 22 |
| --- | --- | --- |
| 6.2 | Bottler Screens | pg. 23 |
| 6.3 | Rinser Settings | pg. 28 |
| 6.4 | Labeler Settings | pg. 30 |
| 6.5 | System Settings | pg. 32 |

SECTION SEVEN – CLEANING CYCLE

REFERENCES

WIRING BLOCK DIAGRAMS

# SECTION ONE – GENERAL INFORMATION

## TERMINOLOGY OF MACHINE

| Micro Filler |  |  |
| --- | --- | --- |
| Item No. | Assembly Name | Description |
| 1 | Conveyor | Moves bottles through system |
| 2 | Filler | Fills bottles |
| 3 | Micro Elevator | Cap Sorter |
| 4 | Cap Chute | Places caps to be picked |
| 5 | Capper | Tightens caps on top of bottles |

## SPECIFICATIONS – STANDARD MACHINE

| ITEM | SPECIFICATION |
| --- | --- |
| CONVEYOR WIDTH | 4.5 INCHES STANDARD |
| MACHINE SPEED | VARIABLE UP TO 32 Bottles per Minute (BPM) |
| OVERALL DIMENSIONS | h: 77 9/16”; L: 98 5/16”; w: 34 5/8” |
| ELECTRIC REQUIREMENTS | 110VAC 6 AMPS |
| AIR REQUIREMENTS | 90 PSI @ 10 CFM |

## FUNCTIONAL DESCRIPTION OF MACHINE

The In-line Micro Filler is a fully automatic in-line carbonated filling machine that can fill various bottle sizes with minimal amounts of waste. This means less product down the drain and more bottles filled.

The machine can be configured as a 4-, 6-, or 8-head machine.

The machine indexes the set target of bottles into position under the fill tubes. The straightener closes on the bottles, further aligning them to the fill tubes, and the filling manifold lowers down onto the bottles. The fill cycle consists of the following: pulling vacuum, charging bottles with CO2, filling bottles to a consistent fill level, removing pressure from the bottle, and releasing the bottles to move down the line. The next group then indexes in and the cycle is repeated.

After filling, the bottles move into the Crowner section of the machine. A crown is placed onto the top of the bottle and then formed over the bottle to provide a sufficient seal. The bottles index one at a time to have the crowns formed.

### Micro Elevator

|  | The Micro Elevator is an automatic cap sorter. Simply load the elevator with caps and it will sort them and place them in the correct orientation for use. Always make sure that the elevator is full so that it will keep up with production. - Hopper – load caps here - Door – allows access to the back - Extension – connects sorter to Chute (pg. 14) - Bracket – connects sorter to Chute - Tensioner – tightens/loosens the chain - Turnbuckle – controls angle of sorter |
| --- | --- |
|  | If a jam ever occurs, simply go to the back of the elevator, rotate the sensor out of the way, rotate the small door lock, and pivot the door to release any jammed caps. Close the door back and rotate the door lock and sensor back into place. If you forget to do this every crown will end up on the floor. |

#### Adjustments

The only adjustment for the Micro Elevator is the angle. The angle should be set to not allow backwards caps or multiples to go all the way up the chain. This angle is adjusted at the Turnbuckle (6) underneath the sorter. Disconnect the Bracket (4) prior to changing the angle of the sorter. Then loosen the nuts on either end of the Turnbuckle and simply rotate the turnbuckle noting the direction of rotation on the sorter.

Jog (pg. 33) the sorter to check the new angle. Once good, tighten both nuts back down.

DO NOT ADJUST THE ANGLE OF THE ELEVATOR WHILE THE BRACKET IS STILL CONNECTED TO THE CAP CHUTE!!!!

### Filler Conveyor

|  | The Conveyor for the Filler controls the indexing of bottles for filling and the positioning of the bottles under the fill tubes. - Entry Gate – creates spacing between cycles - Exit Gate – positions bottles under fill tubes - Straightener – positions bottles under fill tubes (CHANGE PART per bottle) - Back Plate - positions bottles under fill tubes - Photo-eye – counts bottles to start cycle - Railing – keeps bottles on conveyor - Motor – wash down rated |
| --- | --- |

#### Adjustments

Place multiple bottles on the conveyor in different locations and adjust all railing accordingly.

| Entry/Exit Gates (1/2): Adjust the location of the Exit Gate (2) to align the bottles under the fill tubes. Loosen the bolts circled and slide left/right. Push a bottle all the way over to the exit gate (cylinder fully extended) and ensure it is aligned under the far right fill tube. Put full number of bottles in place (4, 6, or 8) and move the Entry Gate (1) into position right in-between bottle in position 1 and the first bottle in the next group. It should not hit the bottles and push them back but just sit in-between the two and not allow the bottles to move into position until the cylinder retracts. |  |
| --- | --- |
| Straightener (3): Jog (pg. 33) the Straightener and check that the bottles are aligned properly under the fill tubes. Move Straightener in/out as needed to align bottles. - Loosen bolt circled and slide assembly in/out as needed. With the Straightener engaged, jog the fill head down to make sure none of the fill tubes hit the bottles. |  |
| Back Plate (4): Adjust the Back Plate to have the bottles centered under the fill tubes. Front to back adjustment is allowed from loosening star-knobs. |  |

#### Change-Over

The only part that changes for different bottles is the Straightener (3). When doing a change-over, just remove the bolt circled above, remove the Straightener assembly, and replace with the new assembly.

### Capper Conveyor

|  | The Conveyor for the Capper controls the indexing of bottles for placing caps on top of bottles and for capping. - Side Belts - moves bottles into position - Hand Wheel - allows for easy changeovers between different bottle diameters - Motor - wash down stepper - Retro-reflector - controls positioning of bottles under capper |
| --- | --- |

#### Adjustments

The only adjustment for this conveyor is the in and out adjustment when changing bottle diameter sizes. This is done by adjusting both Hand Wheels (2) in the same direction. Open the gap between the Side Belts (1) until you can fit the new bottle in-between. Then tighten down onto the bottle until it is snug.

DO NOT OVER TIGHTEN OR THE MOTOR WON’T BE ABLE TO MOVE THE BOTTLES.

BEFORE ADJUSTING IN AND OUT MAKE SURE TO LOOSEN THE HORIZONTAL PLATE ON THE CAP CHUTE!!!!!

### Filler

|  | The Filler is what fills the bottles with product. Cycle: - Bottles index into position - Straightener positions bottles - Fill head comes down onto bottles - Vacuum - Fill - Snift - Everything releases and bottles released to Capper - Inlet – customer connection point - Manifold – distributes product to different heads - Filling Manifold – fills bottles (CHANGE PART per bottle) - Control Box – See Section 6 - Hand Wheel – Adjusts Filling Manifold height for changeover |
| --- | --- |

Temperature and Pressure are critical components to ensure acceptable and repeatable fill levels, reference Zahm & Nagel Chart for Equilibrium, Temperature, and Pressure (Zahm & Nagel).

| Temp Goal: | 1-2 below Equilibrium Temperature |
| --- | --- |
| Pressure Goal: | 1-2 psi above Equilibrium Pressure |

#### Adjustments

The only adjustment is the vertical height of the Filling Manifold (3). This height is adjusted by the Hand Wheel (5).

Place bottles into position, then jog (pg. 33) the fill head down. Adjust the height until the seals are on the bottles. Then jog the CO2 Charge (pg. 33) and go to the Bottler Settings (pg. 24) and make sure that Beer Pressure is being maintained. Adjust until no leaks.

#### Change-Over

DISCONNECT THE AIR & CO2 PRIOR TO PERFORMING A CHANGEOVER!!!

The only change part is the Filling Manifold (3). Unhook all 4 air lines from the manifold. Loosen the shaft collar on the pinch valves and move them out of the way. Then, remove the outside bolts first and then, with a hand holding the manifold up, remove the 2 center bolts. Remove the manifold assembly and replace with the next one.

### Capper

This part of the machine is where the cap is tightened onto the top of the bottle. - Vertical Adjustment – moves the capping assembly up and down for different height bottles - Capping Cylinder – extends to bring cap down to top of bottle - Capping Assembly – rotates the cap to the appropriate rpm - Cap Delivery Assembly – takes cap out of chute and places it in the capping chuck - Capping chuck – tightens the cap onto the bottle - Cap Button – made specific for each cap, used to move cap from chute to capping chuck

#### Capping Process

- Holdback cylinder retracts to allow cap to move to pick-off position

  - Holdback cylinder then moves back out to stop next cap from sliding down once the first one is picked out of the chute

- Shuttle moves cap under the chuck

- Inserter moves the cap into the chuck

- Inserter moves back down

- Shuttle extends back out

- Capping

### Cap Chute

|  | This part of the machine is what places the cap in place to then be put in the chuck. - Chute – places caps in place to be put in chuck - Hand Wheel – adjusts height of chute to right height for cap transfer - Angle Plate – allows for chute angle adjustment to match Micro Elevator (pg. 7) - Horizontal Plate – allows for side to side adjustment to match Micro Elevator - Set Screw – Adjusts placement of cap for transfer |
| --- | --- |

# SECTION TWO – UNCRATING AND INSTALLATION

## 2.1	ELECTRICAL

A grounded electrical male plug is provided with the machine, and is connected to the main electrical enclosure on the rear side of the machine.  Plug this into any grounded receptacle. The main power switch can be found on the right side of the Electrical Enclosure.

## 2.2	COMMUNICATION CABLES

If installed with Rinser and Labeler, connect M12 communication cables between the machines.

## 2.3	PNEUMATIC

The only components that receive unregulated air are the Crowner and the Filling Bar, everything else gets regulated air.

For compressed air, to the right of the Pneumatic Enclosure is an air filter/regulator with a 1/4” male quick disconnect attached.  You can supply compressed air to the machine by either a mating quick disconnect on the end of an air hose, or you can permanently pipe air to the machine using standard pipe and connecting directly into the air filter using threaded pipe connections.  If you permanently pipe into the system we recommend a cut-off valve be mounted at the machine.  Some changeover adjustments are easier if the operator is able to temporarily turn off the air pressure.

|  | The regulator on the left is for air and the one on the right is for CO2. |
| --- | --- |
| Tubing Color Guide: - Blue Top: CO2 Charge - Black Bot: Vacuum - Yellow: Snift - Red: Speed |  |

## 2.4	INSTALLING IN PRODUCTION LINE

Move the machine into its permanent location. Adjust the conveyor height of the machine to match the heights of the adjoining machines as required. Leveling casters are provided with the Micro Filler that allow you some vertical adjustment. Position the conveyor ends as close to each other as possible and then use conveyor rails to guide the containers across narrow dead plates onto the conveyor.

## 2.5	LEVELING THE BASE MACHINE

Once the machine is installed, level the main conveyor through the machine by using a bubble level.  Place the bubble level along the length of the machine. Leveling the machine is important to the flow of the line as it allows for more seamless transitions between machines. The squaring and straightness of the base machine will ensure the machine operates correctly.

# SECTION THREE – OPERATIONAL ADJUSTMENTS

## 3.1	ADJUSTING FILL LEVELS

Filling on the Micro Filler is controlled by flow meters on each line. This gives the ability to adjust the fill levels of each head individually. This is controlled in the PLC and can be adjusted on the Filling Set Points page of the Touchscreen (pg. 23).

Each pinch valve closes when the product dispensed reaches the target value set on the Filling Set Points screen.

## 3.2	FOAM ISSUES

There are 2 factors that will affect whether or not too much foaming occurs; temperature and pressure.

Temperature: Product foams during fill

Pressure: Product foams over as soon as fill heads lift

# SECTION FOUR – PERIODIC MAINTENANCE, CLEANING, AND LUBRICATION

## 4.1	MAINTENANCE

Ensure that you perform a monthly visual inspection for wear on the Filling Manifold seals, conveyor chain, Side Belts, and Crowning Head.

## 4.2	CLEANING THE MACHINE

The Micro Filler comes in stainless and aluminum construction. Cleaning the machine regularly is recommended.

Hook up at the Customer Connection point and run the Cleaning cycle on the Systems Settings page.

- Load bottles under the filling head, jog straightener in, lower filling heads onto bottle.

- Prepare cleaning solutions, (HOT CAUSTIC RECOMMEND 160 DEGREES 170 MAX.)

- From the system menu select the cleaning time per stage (speed, snift, vacuum paths).

- Connect cleaning solution product supply manifold.

- Select Cleaning ON. Switch to the monitoring menu to observe progress.

- When the vacuum valve opens remove the tube from the vacuum pump to the tank to fill the tanks with cleaning solution, then drain the tank.

- Repeat hot water through the machine, same process with sanitizer prior to fill

  - Pressure not to exceed 25psi

Wash down conveyors and components with hot water.

## 4.3	LUBRICATION

The only lubrication points on the machine are:

- The conveyor idler sprockets inside the frame of the machine need grease quarterly. The conveyor idler sprockets are located under the conveyor chain.

- Any threaded rod for linear motion should have light oil (food safe) applied to it periodically to prevent rust and to keep the mechanism moving freely.

- Flange mount bearings; these have grease fittings and should be lubricated quarterly

# SECTION FIVE - TROUBLESHOOTING

The list below represents a few scenarios in which troubleshooting may need to occur.

## 5.1	NOTHING WORKS AT ALL/POWER IS ON BUT NOTHING WORKS

- Check main power.  Is machine plugged in?  Is main power switch turned on?

- Check fuses inside control panel.

- Are conveyor speed controls turned up above zero (in the Touchscreen)?

## 5.2	OPERATIONAL INCONSISTENCIES (NOTHING IS BEING FILLED)

- Confirm that filling is on and that the counting eye is changing states between bottles.

- Confirm that air supply is on.

- Confirm bottles per cycle is not set to zero.

- Confirm CO2 pressure is set 5-10psi above tank head pressure.

# SECTION SIX – SCREENS

## Main Screen

The Main Screen has three subsections: Bottler Settings, Rinser Settings, and Labeler Settings. The commonly edited items for each machine are listed on this screen. Below each subsection are screen buttons that take you to specific screens. Bottler screens are blue, Rinser screens are green, and Labeler screens are red.

A batch count is carried in the top right corner and a reset button is there to reset the count back to zero.

There are multiple jog buttons throughout each screen. Each one will have a stated status. The stated status is what the status of each function is. If a certain function is desired to run, then stated status must say on.

Example: The button that says Filling On indicates that the filling function is on. If it said Filling Off then the filling function would be turned off.

## 6.2	Bottler Screens

### 6.2.1	Filling Set Points

Each filling head (numbered from left to right) has its own set point target. This allows for dialing in to specific fill levels on each head. For the average twelve ounce bottle, a setting between 1000 and 1050 should yield a full twelve ounces.

When a cycle is running, each of the vertical bars will begin to fill in to show the progress of each fill head. When the filling has reached its set point, the color on the bar will change from a dark red to bright red. The last bottle to fill will remain a darker red to show which fill head has the slowest fill.

Below each bar is a Jog button. This button allows liquid to flow through the specified fill head to ensure each one is working properly or to remove any foam that may be present in the line. This allows for the user to go one by one and check that each one is good. However, in the top right there is a Jog All Beer Valves button that will open each fill head at once.

This should be used at the beginning to push product through, cool down all lines, and push any foam through the lines. It should also be used at the end of each run to allow for cleaning.

Also in the upper right corner is the Finish Cycle button, which should be used when filling is in progress and the machine has stopped. When the Finish Cycle button is pressed, each fill head and the speed valve will close, and the machine will go on into the snift cycle to completion.  This lets you move on rather than stop the machine and clear out bottles.

### 6.2.2	Bottler Settings

| Current Count | Displays the current count of bottles as the filling cycle is occurring |
| --- | --- |
| Sequence Status | Displays the current state in the cycle |
| CO2 Charge Offset | The machine takes the measurement of the beer pressure and adds to it the CO2 offset for the purpose of charging the bottles. So if the beer pressure is 14psi, then with a setting of -1.0 the bottle would stop purging at 13psi. If the offset is positive, then the bottles will pressurize above the beer pressure and when the valves open, the beer will be forced upward into the tube which will create more foam. |
| Snift Target | The low psi target that must be reached before the filling heads will lift from the bottles at the end of the cycle. This setting should allow the beer to foam in the neck of the bottle but not so aggressively that beer is lost over the top. A slight puff sound is what is desired as the heads lift. |
| Filling On/Off | If off, the Filling Manifold will never come down and engage |
| Sorter On/Off | Turns the sorter on/off |
| Pulse On/Off | At the end of the snift cycle, the pulse function will disturb the beer by jetting CO2 into the top of the bottle in an attempt to produce foam. There is a delay and dwell in the Timer screen. |
| Vacuum On/Off | Turns the Vacuum function (including pump) on or off |
| Tank Control | Turns the tank pressure control on or off. The tank pressure is displayed on the screen and the target setting is entered just below that. This setting should be adjusted so that the tank pressure measured at your tank is close to 15, if that is the max pressure your tank can hold. |
| Pre-Evac x 2 | Forces the vacuuming and CO2 purge of the bottle to be done twice before filling. The settings for vacuum length and CO2 target are used for both cycles. |
| Crowning On/Off | Turns the Crowning Head on/off If off, the bottles will not stop in the Side Belts |
| Finish Cycle | This button completes the filling cycle and forces the machine into the snift cycle. |
| Jog All Beer Valves | As stated above, pushing this button will open all beer lines at once. |

Beer Pressure and Off-Gas pressure are continuously displayed as the machine is running.

Each of the lower buttons takes you to a different screen.

### 6.2.3	Filling Timers

| Fill Heads Down Delay | A delay after the straighteners have clamped the bottles and the downstream photo-eye has cleared before the fill heads go down into the bottles. |
| --- | --- |
| Fill Start Delay | A delay after the fill heads are down but before the entire filling cycle starts. |
| Entry Gate In Delay | After the fill heads are down, a delay before the entry gate cylinder extends out. If extended too soon it might prevent the full number of bottles from moving into position under the fill heads. |
| Entry Gate Out Delay | A delay before the entry gate cylinder retracts. If done too soon, bottles might get past the Exit Gate before it extends. |
| Exit Gate Open Delay | A delay after the straightener has released the completed bottles before the cylinder retracts. |
| Exit Gate Open Dwell | The length of time the exit gate remains open to allow the full group of bottles to pass before extending back out. |
| CO2 Charge Offset | The machine takes the current reading of beer pressure and adds/subtracts the offset to set a target psi that the bottles will be charged to before opening the beer valves. |
| Vacuum Delay | A delay after the filling cycle is started before the vacuum valve opens to evacuate the atmosphere in the bottles. |
| Vacuum Time | The length of time the vacuum valve is open to evacuate the bottles. |
| Beer Start Delay | After the bottles are vacuumed and then charged, a delay to start filling |
| Speed Valve Delay | A delay after beer starts before the speed valve opens to release gas. |
| Snift Valve Delay | A delay after all bottles have filled before the snift valve opens |
| Snift Target (psi) | The target bottle pressure that snifting will achieve before the heads lift |
| Vacuum Pump Time | The length of time the vacuum pump runs each cycle to replenish the vacuum used to evacuate the bottles |
| Purge Lines Delay | A delay after fill heads go up before the educator begins to run in order to purge the snift lines. |
| Purge Lines Time | The length of time the educator runs to purge lines |
| CO2 Pulse Delay | If pulse function is turned on, the delay after snifting has finished before the pulse function begins to create foam in the neck. |
| CO2 Pulse Length | The length of time the pulse function occurs |
| Crown Head Delay | After the Side Belts have stopped with the bottle in position, a delay before the Crowning Head goes down. |
| Crown Head Length | The length of time the Crowning Head is down to complete the crowning function. |
| Straightener Delay | After the bottles are fully counted before the straightener comes in to clamp them |

Each of the lower buttons takes you to a different screen.

## 6.3	Rinser Settings

| Rinsing On/Off | Turns the rinse function on/off |
| --- | --- |
| Rinser OK | Displays the current status of the Rinser |
| Backup CTRL | This utilizes the backup control photo-eye to prevent the Rinser from over-running the need for rinsed bottles. |

The bottom six buttons on the left are for jogging the devices of the Rinser while the machine is not running, to test the functions.

| Forward and Reverse Speed | This is the speed of the pivot arm back and forth |
| --- | --- |
| Start and Forward Position | This is the position the pivot arm starts at and for the rinse. 8000 is 180 degrees. |
| Bottles per Cycle | This should match the number of tubes on the Manifold |
| Backup Delay | The length of time the backup photo-eye is blocked to pause the Rinser from releasing the next group of bottles |
| Pivot FWD delay | The delay after the bottles are clamped before the arm moves |
| Pivot Back Delay | The delay after the bottles have been drained before the arm returns home |
| Holdback Close Delay | The delay before the holdback gate closes after counting the bottles per cycle |
| Holdback Open Delay | The delay after the exit gate has opened before the entry gate opens |
| Exit Gate Open Delay | The delay after the clamp releases the bottles to when the exit gate opens |
| Exit Gate Open Time | The length of time the exit gate is open for the full group of bottles to pass by |
| Water Pump Delay | The delay after bottles are inverted until the rinse begins |
| Water Pump Dwell | This is the rinse time for how long the solution is in the bottles |
| Drain Time | The length of time after the rinse finishes for the bottles to drain before being returned to the conveyor |

Each of the lower buttons takes you to a different screen.

## 6.4	Labeler Settings

| Label On/Off | Turns the labeling function On/Off |
| --- | --- |
| Jog Label | Press to jog one label through the machine when machine is not running |
| Backup Ctrl On | Turns on or off the backup control function, which starts and stops the Spacing Wheel based on the backup photo-eye being blocked or not |
| Spacing On | Turns on or off the spacing function |
| Wrap Belt On | Turns on or off the wrap belt |
| Date Code On or Off | Turns on or off the date coder function, which will send a signal to the coder |
| Code Signal | Can select to send a signal to the coder at the start of the label (thermal transfer or inkjet coders) or at the end of the label (Hotstamp coders) |
| Gap Error OK | If a gap error occurs where the machine begins to feed a label but sees no gap, then the machine will shut down and the error will display. Press the Stop Button or Error Reset button on the screen to clear the error |
| Label Error OK | If the label sensor malfunctions or no labels are present in the sensor, then the machine will timeout and stop after the label error is complete. This button will display that the error has happened. Press the Stop Button or Error Reset button to clear the error |
| Labeler OK | If the labeler has paused from backup, this will display the status |
| Error Reset | Clears the Label Error and the Gap Error |
| Label Start Delay | After the bottle is detected, the delay before the label begins feeding. This is in inches of movement of the conveyor or the wrap belt |
| Label Stop Delay | This is the delay for the label to stop feeding after a gap between labels has been detected by the gap sensor. This is in inches of label feed |
| Label Ratio | This determines the speed of the label feeding versus the wrap belt. A ratio of 1.0 would mean the label speed is matching. A lower number slows the label feed and a larger number increases it |
| Wrap Belt Ratio | This number determines the ratio of the wrap belt to the main speed of the conveyor. A lower number slows it down and a higher number increases it |
| Conveyor Speed | This is the speed of the conveyor in inches per minute. This sets the main speed of the machine. |
| Backup Delay | When the backup sensor just after the wrap belt is blocked. A delay before the Spacing Wheel pauses to prevent more bottles from entering the machine |
| Code Start Delay | A delay before the signal to initiate a code is sent. This works in conjunction with the Code Signal Start button being set to “Start” or “End” |
| Code Signal Dwell | This is the length of the signal sent to the coder. For most devices, a value of 10 is more than enough |
| No Gap Timeout | After the label begins feeding, the machine is looking for the next gap between labels. If it does not see one in this time frame (a value of 500 is equal to 5 seconds), it will stop the machine and stop feeding labels |
| No Label timeout | If the machine is running and does not see any labels in the gap sensor, the machine will stop after this time (again, a value of 500 is equal to 5 seconds) |

Each of the lower buttons takes you to a different screen.

## 6.5	System Settings

| Auto Mode On/Off | Places machine in run mode for all components |
| --- | --- |
| Cleaning On/Off | Turn this on to initiate cleaning functions |
| Step Belts On/Off | Turns the Side Belts on/off |
| Hot Water On/Off | Turns Hot Water/CO2 Inserter on/off |
| Filler Conveyor Speed | This sets the speed of the conveyor and is in inches per minute. Maximum of 1200 |
| Sorter Belt Speed | This sets the speed of the Micro Elevator belt |
| Stepper Belt Speed | This sets the speed of the Side Belts |
| Bottles per Cycle | This is the number of bottles the Filler will be counting as they enter |
| Step Belt Delay | This is the time that the Side Belts will continue to move after breaking the crowning photo-eye before they stop. Used for positioning the bottle under the Crowning Head |
| Hot Water Delay | This is the lag time from when the photo-eye is blocked to when the Hot Water Insert is triggered. |
| Cleaning Cycle Time | This is the time for the cycle to run; this number is in seconds 300 = 5 minutes |
| Hot Water Dwell | This is how much time the Hot Water runs prior to turning back off |

Each of the lower buttons takes you to a different screen.

### 6.5.1	Jog Devices

Each button will jog a specified function. This is a very useful screen for setup and troubleshooting purposes.

Each of the lower buttons takes you to a different screen.

### 6.5.2	Check Counters

This screen allows you to check the counter functions to ensure they are working properly.  You can jog each head individually and see the counter moving.  Or during operation you can see the values change as each cycle occurs.  A counter not counting up either has a connection issue or has failed.

Each of the lower buttons takes you to a different screen.

### 6.5.3	Startup Checklist

Use this checklist prior to starting a run.

Each of the lower buttons takes you to a different screen.

# SECTION 7 – CLEANING CYCLE

- Lower heads onto bottles

- Connect sanitizer

- Open beer valves

- Open speed valve for 2 minutes

- Close speed valve

- Open snift valve for 2 minutes

- Close snift valve

- Open vacuum valve for 2 minutes

  - Remove top tube on vacuum tank, when sanitizer comes out, open bottom valves

- Flush with hot water at the end of the cycle

# References

Zahm & Nagel. "Volumes of CO2 Gas Dissolved in Water." Www.zahmnagel.com. N.p., Apr.

2016. Web. 4 Jan. 2017.

WIRING BLOCK DIAGRAMS.